Computer Architecture Project Report

1. Introduction

This project implements a basic 16-bit computer architecture capable of executing 11 instructions. The design follows the classical Morris Mano basic computer model, built and simulated at the component level.

The instruction word is 16 bits wide, structured as follows:

| Bit 15 | Bits 14–12 | Bits 11–0 |
| --- | --- | --- |
| I (Indirect bit) | Opcode (3 bits) | Address (12 bits) |

2. Instruction Set

2.1 Memory Reference Instructions (MRI)

These instructions operate on memory using the 12-bit address field.

| Mnemonic | Opcode | Operation |
| --- | --- | --- |
| AND | 000 | AC ← AC ∧ M[AR] |
| ADD | 001 | AC ← AC + M[AR] |
| LDA | 010 | AC ← M[AR] |
| BUN | 011 | PC ← AR |

2.2 Register Reference Instructions (RRI)

These instructions operate directly on registers. The indirect bit I = 0 and opcode = 111, with a unique bit pattern in the address field identifying each instruction.

| Mnemonic | Operation |
| --- | --- |
| CLA | AC ← 0 |
| CMA | AC ← AC' |
| CIR | Circular shift AC right through E |
| CIL | Circular shift AC left through E |
| INC | AC ← AC + 1 |
| SPA | Skip next instruction if AC > 0 |
| SNA | Skip next instruction if AC < 0 |

3. Components

| Component | Size | Role |
| --- | --- | --- |
| RAM | 4096 × 16-bit | Stores instructions and data |
| AR (Address Register) | 12-bit | Holds the memory address for read/write |
| PC (Program Counter) | 12-bit | Holds the address of the next instruction |
| DR (Data Register) | 16-bit | Temporarily holds data fetched from memory |
| AC (Accumulator) | 16-bit | Primary register for all arithmetic and logic operations |
| IR (Instruction Register) | 16-bit | Holds the currently fetched instruction |
| Bus | 16-bit | Shared data path connecting all components |
| ALU | 16-bit | Executes AND, ADD, complement, and shift operations |
| Sequence Counter | — | Generates timing signals T0 through Tn to control micro-operations |
| Instruction Decoder | 3-to-7 | Decodes the 3-bit opcode to activate the appropriate instruction logic |

4. Memory Unit (RAM)

Specifications

| Property | Value |
| --- | --- |
| Size | 4096 × 16-bit |
| Address Input | AR_OUT (12-bit) |
| Data Output | MEM_OUT (16-bit) |
| Operation | Read Only |

Control Logic

RAM read is activated under three conditions:

Read = T1 + (D7' · I_BIT · T3) + (T4 · (D0 + D1 + D2))

| Condition | Purpose |
| --- | --- |
| T1 | Fetch instruction from memory |
| D7' · I_BIT · T3 | Indirect cycle — fetch effective address |
| T4 · (D0 + D1 + D2) | Execute cycle — fetch operand for AND, ADD, LDA |

Micro-operations

| Timing | Condition | Operation |
| --- | --- | --- |
| T1 | — | IR ← M[AR] |
| T3 | I=1, D7' | AR ← M[AR] |
| T4 | D0+D1+D2 | DR ← M[AR] |

5. Address Register (AR)

Specifications

| Property | Value |
| --- | --- |
| Size | 12-bit |
| Input | BUS_OUT (bits 0–11) |
| Output | AR_OUT (12-bit) |
| Control | CLK, ld |

Control Logic

AR_load = T0 + T2 + (T3 · D7' · I_BIT)

| Condition | Operation |
| --- | --- |
| T0 | AR ← PC (fetch cycle) |
| T2 | AR ← IR(0–11) (decode cycle) |
| T3 · D7' · I_BIT | AR ← M[AR] (indirect cycle) |

6. Program Counter (PC)

Specifications

| Property | Value |
| --- | --- |
| Size | 12-bit |
| Input | BUS_OUT via MUX, PC_INC |
| Output | PC_OUT (12-bit) |
| Control | CLK, ld |

Control Logic

Increment condition:

PC_INC = T1 + (T3 · (B3 + B4) · D7 · I')

Load condition:

PC_load = T1 + (T3 · ((B3*AC[15]’) + (B4*AC[15])) · D7 · I') + (T4 · D4)

| Condition | Operation |
| --- | --- |
| T1 | PC ← PC + 1 (fetch cycle) |
| T3 · (B3+B4) · D7 · I' | PC ← PC + 1 (skip — SPA/SNA satisfied) |
| T4 · D4 | PC ← AR (BUN instruction) |

MUX Role

The MUX selects between BUS_OUT (for load via BUN) and PC+1 (incremented value) before feeding into the PC register.

7. Data Register (DR)

Specifications

| Property | Value |
| --- | --- |
| Size | 16-bit |
| Input | BUS_OUT |
| Output | DR_OUT (16-bit) |
| Control | CLK, ld |

Control Logic

DR_load = T4 · (D0 + D1 + D2)

| Condition | Operation |
| --- | --- |
| T4 · D0 | DR ← M[AR] (AND operand fetch) |
| T4 · D1 | DR ← M[AR] (ADD operand fetch) |
| T4 · D2 | DR ← M[AR] (LDA operand fetch) |

8. Accumulator (AC)

Specifications

| Property | Value |
| --- | --- |
| Size | 16-bit |
| Input | ALU_OUT |
| Output | AC_OUT |
| Control | CLK, ld, clr |

Control Logic

AC_load:

AC_load = (T3 · I' · (B9 + B5) · D7) + (T5 · (D0 + D1 + D2))

AC_clr:

AC_clr = (T3 · D7 · B11 · I')

| Condition | Operation |
| --- | --- |
| T3 · I' · B9 · D7 | AC ← 0 (CLA) |
| T3 · D7 · B11 · I' | AC ← 0 (SNA — clear condition) |
| T3 · I' · B5 · D7 | AC ← AC' (CMA) |
| T5 · D0 | AC ← AC ∧ DR (AND) |
| T5 · D1 | AC ← AC + DR (ADD) |
| T5 · D2 | AC ← DR (LDA) |

9. Instruction Register (IR)

Specifications

| Property | Value |
| --- | --- |
| Size | 16-bit |
| Input | BUS_OUT |
| Output | IR_OUT (16-bit) |
| Control | CLK, ld |

Control Logic

IR_load = T1

| Condition | Operation |
| --- | --- |
| T1 | IR ← M[AR] (fetch instruction from memory) |

10. Bus System

Specifications

| Property | Value |
| --- | --- |
| Type | 16-bit Multiplexed Bus |
| Inputs | AR_OUT, PC_OUT, IR_OUT, MEM_OUT |
| Output | BUS_OUT |
| Select Lines | S2, S1, S0 (3-bit) |

MUX Selection

| S2 | S1 | S0 | BUS_OUT |
| --- | --- | --- | --- |
| 0 | 0 | 0 | AR_OUT |
| 0 | 0 | 1 | PC_OUT |
| 0 | 1 | 0 | IR_OUT |
| 0 | 1 | 1 | MEM_OUT |

Select Line Equations

S2 = (D7' · T3 · I_BIT) + T1 + (T4 · (D0 + D1 + D2)) + T2

S1 = (D7' · T3 · I_BIT) + T1 + (T4 · (D0 + D1 + D2)) + T0

S0 = (D7' · T3 · I_BIT) + T1 + (T4 · (D0 + D1 + D2)) + T2 + (T4 · D4)

11. Arithmetic Logic Unit (ALU)

Specifications

| Property | Value |
| --- | --- |
| Inputs | AC_OUT, DR_OUT |
| Output | ALU_OUT |
| Select Lines | a2, a1, a0 (3-bit MUX) |
| Operations | CMA, INC, AND, ADD, CIR, CIL, Pass DR |

ALU Operations

| MUX Input | Operation | Result |
| --- | --- | --- |
| CMA_AC | Complement AC | AC' |
| INC_AC | Increment AC | AC + 1 |
| M_AND | AND | AC ∧ DR |
| M_ADD | Add | AC + DR |
| DR_OUT | Pass DR | DR |
| CIR | Circular Shift Right | AC >> 1 |
| CIL | Circular Shift Left | AC << 1 |

Select Line Equations

a2 = (D0 · T5) + (T3 · I_BIT' · B7 · D7) + (T3 · I_BIT' · B6 · D7) + (D2 · T5)

a1 = (T3 · I_BIT' · D7 · (B5 + B7 + B6)) + (D1 · T5)

a0 = (T3 · I_BIT' · D7 · (B9 + B6)) + (T5 · (D1 + D2))

12. Sequence Counter (SC)

Specifications

| Property | Value |
| --- | --- |
| Output | T0 – T6 (via 3-to-7 decoder) |
| Control | CLK, SC_CLR |

Control Logic

SC resets to T0 when SC_CLR is asserted.

SC_CLR = (D7 · I_BIT' · T4) + T6 + (T6 · (D0 + D1 + D2)) + (T5 · D4)

| Condition | Reason |
| --- | --- |
| D7 · I_BIT' · T4 | RRI instructions complete at T4 |
| T6 | Maximum timing cycle reached |
| T6 · (D0 + D1 + D2) | MRI instructions complete at T6 |
| T5 · D4 | BUN completes at T5 |

13. Instruction Decoder (Opcode Decoding)

Specifications

| Property | Value |
| --- | --- |
| Input | IR_OUT (16-bit) |
| Opcode Output | D0 – D4, D7 (3-to-8 decoder) |
| Address Bits Output | B3, B4, B5, B6, B7, B9, B11 |
| Control | Enable = 1 |

IR Decoding

IR_OUT is split into three fields:

| Field | Bits | Signal |
| --- | --- | --- |
| Indirect bit | IR[15] | I_BIT |
| Opcode | IR[14–12] | OPCODE |
| Address field | IR[11–0] | ADD_FLD |

Opcode Decoder Output

| D | Instruction |
| --- | --- |
| D0 | AND |
| D1 | ADD |
| D2 | LDA |
| D4 | BUN |
| D7 | RRI (Register Reference Instruction) |

Address Field Bits for RRI

| Bit | RRI Instruction | Full Form |
| --- | --- | --- |
| B11 | CLA | Clear Accumulator |
| B9 | CMA | Complement Accumulator |
| B7 | CIR | Circular Shift Right |
| B6 | CIL | Circular Shift Left |
| B5 | INC | Increment Accumulator |
| B4 | SPA | Skip if Plus Accumulator |
| B3 | SNA | Skip if Negative Accumulator |

14. Output Window

The Output Window provides real-time visibility into the state of all major signals and registers during simulation.

Register Outputs

| Signal | Width | Displays |
| --- | --- | --- |
| MEM_OUT | 16-bit | Current memory data |
| AR_OUT | 12-bit | Address register value |
| PC_OUT | 12-bit | Program counter value |
| DR_OUT | 16-bit | Data register value |
| AC_OUT | 16-bit | Accumulator value |
| IR_OUT | 16-bit | Current instruction |

Timing Signals

| Signal | Purpose |
| --- | --- |
| T0 – T6 | Active timing step indicator |

Decoded Instruction Signals

| Signal | Purpose |
| --- | --- |
| D0 – D7 | Active instruction decoder output |

Address Field Bits

| Signal | Purpose |
| --- | --- |
| B2, B3, B4, B5, B6, B7, B9, B11 | Active RRI instruction bit |

Clock

| Signal | Purpose |
| --- | --- |
| CLK | Master clock pulse driving all registers and sequence counter |

15. Conclusion

- Implements a 16-bit basic computer capable of executing 11 instructions — MRI (AND, ADD, LDA, BUN) and RRI (CLA, CMA, CIR, CIL, INC, SPA, SNA).

- All components (RAM, AR, PC, DR, AC, IR, Bus, ALU, Sequence Counter, Instruction Decoder) designed and simulated at gate level with precise control logic equations.

- Fetch, decode, and execute cycles correctly sequenced through timing signals T0–T6.

- Sequence Counter resets automatically upon completion of each instruction via SC_CLR.

- 3-bit MUX Bus efficiently routes data between all components.

- ALU handles all arithmetic and logic operations, feeding results directly into AC.

- Output Window validates correct real-time operation of all registers, timing signals, decoded instruction outputs, and RRI address bits.